Circuit Analysis (Lab10)

Experiment 9: Verification of Thevenin’s Theorem

Name: …………………………….   Registration No: ………………………...

Date: …………………………….     Grade and Signature: ……………………

| CLO2 CLO3 | Use with identification the circuit components, breadboards, multimeter, power supplies, signal generators, and oscilloscopes. Make measurements in their own constructed electric circuits. |  |  |  |
| --- | --- | --- | --- | --- |
| Psychomotor/Affective | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) |
| Report Marks (3) |  |  |  |  |
|  | Total marks (10) |  |  |  |

Objective

The objective of this lab is verify Thevenin’s theorem and to find the full load current.

Apparatus Required

| Sr. no | Apparatus | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply | Standard | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter | Standard | 1 |
| 5 | Breadboard & Wires | Standard | As Required |
| 6 | Resistors | 1k, 330 ohms | 1,3 |
| 7 | Potentiometer | Standard | 1 |

Description

Statement:

Any linear bilateral, active two terminal network can be replaced by an equivalent voltage source

(VTH). Thevenin’s voltage or V OC in series with looking pack resistance RTH.

Precautions:

- Voltage control knob of RPS should be kept at minimum position.

- Current control knob of RPS should be kept at maximum position

Procedure:

- Connections are given as per the circuit diagram.

- Set a particular value of voltage using RPS and note down the corresponding ammeter readings. To find VTH 3. Remove the load resistance and measure the open circuit voltage using multimeter (VTH). To find RTH

- To find the Thevenin’s resistance, remove the RPS and short circuit it and find the RTH using multimeter.

- Give the connections for equivalent circuit and set VTH and RTH and note the corresponding ammeter reading. 6. Verify Thevenins theorem.

Circuit - 1: To find load current

To find VTH

To find RTH

Thevenin’s Equivalent circuit:

Thevenin Practical Values

|  | E(V) | VTH(V) | RTH(ohms) | IL (mA) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Circuit - I | Equivalent Circuit |
| Values | 12 V | 6 V | 495 ohm | 4.01 mA | 4.01 mA |

Post Lab Questions

  - What is Thevenin’s Theorem?

Thevenin’s theorem states that any linear bilateral two-terminal network can be replaced by an equivalent circuit consisting of a single voltage source called Thevenin voltage (VTH) in series with an equivalent resistance called Thevenin resistance (RTH). This equivalent circuit provides the same output behavior as the original circuit.

  - What are the applications of Thevenin’s Theorem?

Thevenin’s theorem is used to simplify complex electrical circuits into simple equivalent circuits. It is useful in analyzing load current and voltage, studying power systems, troubleshooting electrical networks, and designing electronic circuits. It also helps in understanding the effect of changing load resistance without repeatedly solving the entire circuit.

  - What are the theoretical and practical limitations of Thevenin’s Theorem?

Thevenin’s theorem is applicable only to linear bilateral networks and cannot be directly applied to nonlinear components such as diodes and transistors in nonlinear operation. In practical circuits, measurement errors, internal resistance of instruments, and component tolerances may cause small differences between theoretical and practical results. The theorem also becomes less convenient for circuits containing dependent sources unless special analysis methods are used.