Circuit Analysis (Lab11)

Experiment 10: Verification of Norton’s Theorems

Name: Muhammad Zuhaib              Registration No: 2025-CE-120

Date: 17 May 2026                            Grade and Signature: ……………………

| CLO2 CLO3 | Use with identification the circuit components, breadboards, multimeter, power supplies, signal generators, and oscilloscopes. Make measurements in their own constructed electric circuits. |  |  |  |
| --- | --- | --- | --- | --- |
| Psychomotor/Affective | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) |
| Report Marks (3) |  |  |  |  |
|  | Total marks (10) |  |  |  |

Objective

The objective of this lab is verify Nortan’s theorem and to find the full load current.

Apparatus Required

| Sr. no | Apparatus | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply | Standard | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter | Standard | 1 |
| 5 | Breadboard & Wires | Standard | As Required |
| 6 | Resistors | 1k, 330 ohms | 1,3 |
| 7 | Potentiometer | Standard | 1 |

Description

Statement:

Any linear circuit can be simplified to an equivalent circuit consisting of a single current source and parallel resistance that is connected to a load.   Precautions:

- Voltage control knob of RPS should be kept at minimum position.

- Current control knob of RPS should be kept at maximum position

Procedure for Norton’s Theorem:

- Connections are given as per circuit diagram.

- Set a particular value in RPS and note down the ammeter readings in the original circuit.

To Find IN:

- Remove the load resistance and short circuit the terminals.

- For the same RPS voltage note down the ammeter readings.  To Find RN:

- Remove RPS and short circuit the terminal and remove the load and note down the resistance across the two terminals.  Equivalent Circuit:

- Set IN and RN and note down the ammeter readings.

- Verify Norton’s theorem.

To find load current in circuit 1:

To find IN

To find RN

Norton’s equivalent circuit

Norton’s Practical Values

|  | E (volts) | IN (mA) | RN (ohms) | IL (mA) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Circuit - I | Equivalent Circuit |
| Practical Values | 12 V | 12.1 mA | 495 ohm | 4.01 mA | 4.01 mA |

Post Lab Questions

1. What is Nortan’s Theorem?

Norton’s theorem states that any linear bilateral two-terminal network can be replaced by an equivalent circuit consisting of a current source called Norton current (IN) connected in parallel with an equivalent resistance called Norton resistance (RN). This equivalent circuit behaves the same as the original circuit at the output terminals.

2. What are the applications of Nortan’s Theorem?

Norton’s theorem is used to simplify complex electrical circuits into simple equivalent circuits for easier analysis. It is useful in analyzing load current, studying power systems, troubleshooting electrical networks, and simplifying repeated calculations when load resistance changes. It is also widely used in electronic circuit design and network analysis.

3.  What are the theoretical and practical limitations of Nortan’s Theorem?

Norton’s theorem is applicable only to linear bilateral circuits and cannot be directly applied to nonlinear devices such as diodes and transistors operating in nonlinear regions. In practical circuits, component tolerances, internal resistance of measuring instruments, and wiring errors may produce slight differences between theoretical and practical results. The theorem also becomes more difficult to apply in circuits containing dependent sources unless additional analysis techniques are used.