Circuit Analysis (Lab # 2)

Experiment 2: Measurements of Waveforms and Fundamental Quantities, Basic resistive circuits.

Name: Muhammad Zuhaib	 	                      Registration No:  2025-CE-120

Date: 15-03-2026 	 	           Grade and Signature: ………………………

| CLO3 | Make measurements in their own constructed electric circuits. |  |  |  |
| --- | --- | --- | --- | --- |
| Psychomotor/Affective | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) |
| Report Marks (3) |  |  | Total marks (10) |  |

_______________________________________________________________________________

_

Objectives: This experiment will provide exposure to measurements of waveforms and Fundamental Quantities, Basic resistive circuits. A primary purpose of this lab course is for you to master the use of electronic test equipment and measurement of fundamental quantities.

Equipment required:

Power Supply, Breadboard, Oscilloscope, Signal Generator Connecting wires, Resistances, Multimeter. Procedure

# Experiment # 1

SIGNAL GENERATOR AND OSCILLOSCOPE

- Use the signal generator and the oscilloscope to perform the following tasks.

- Build circuit ‘A’ shown below in Figure 1.

- Set the signal generator to generate a sinusoidal signal with a frequency of 100 Hz and peak-to peak voltage of 5V.

- Set up one probe across input signal, and another across R1 and R2 alternatively.

- Measure and draw all the voltages and time related parameters on the graphs below.

Figure 1 Circuit ‘A’

| Resistor | Input voltage | RMS Voltage | P-P Voltage |
| --- | --- | --- | --- |
| R1 | 5V | 1.07V | 3.04V |
| R2 | 5V | 0.70V | 1.96V |

## Figure 2 Input Voltage Signal 	Figure 3 Voltage Signal across R1

| Figure 5 Voltage Signal across R2 | Figure 6 Draw 7V Sinusoidal |
| --- | --- |

# Experiment # 2

SIGNAL GENERATOR AND OSCILLOSCOPE

- Use the signal generator and the oscilloscope to perform the following tasks.

- Build circuit ‘B’ shown below in Figure 7.

- Set the signal generator to generate a sinusoidal signal with a frequency of 200 Hz and peak-to peak voltage of 7V.

- Set up one probe across input signal, and another across R1, R2 and R3 alternatively.

|  |  | Figure 7 Circuit ‘B’ |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Resistor | Input voltage |  | RMS Voltage |  | P-P Voltage |
| R1 | 7V |  | 0.89V |  | 2.51V |
| R2 | 7V |  | 0.62V |  | 1.76V |
| R3 | 7V |  | 0.96V |  | 2.73V |
| R1 and R2 | 7V |  | 1.51V |  | 4.27V |
| R2 and R3 | 7V |  | 1.59V |  | 4.49V |

- Measure and draw all the voltages and time related parameters on the graphs below.

Figure 8 Input Voltage Signal 	Figure 9 Voltage Signal across R1

Figure 10 Voltage Signal across R2                                                	Figure 11 Voltage Signal across R3

Figure 12 Voltage Signal across R1 and R2 	Figure 13  Voltage Signal across R2 and R3

Post Lab Questions

- Define Ohm’s law.

Ohm’s law states that the current flowing through a conductor is directly proportional to the voltage across it and inversely proportional to the resistance.

V=IR

- What is resistor?

A resistor is a passive electronic component that opposes the flow of electric current in a circuit and is     used to control voltage and current levels.

- How resistor works?

A resistor works by converting electrical energy into heat while limiting current flow. The resistance causes electrons to collide with atoms inside the material, reducing the current.

- What is your inference from the input and output waveforms on oscilloscope?

The oscilloscope shows that the voltage across resistors in a series circuit follows the voltage divider rule. The waveform shape remains sinusoidal but the amplitude decreases according to resistance values.

- Show calculations of 7V input signal (Figure 10) from signal generator.

  - Voltage calculations (Following volt/dev and number of boxes of oscilloscope)

  - Frequency/Period calculations (Following Steps in frequency measurements section)

  - Current calculation (Ohm Law)

#### Voltage:

Vp=3.5V
Vrms=3.52=2.47V

#### Frequency:

f=200Hz
T=1f=0.005s

#### Current using Ohm's Law:

Total resistance:

RT=13.1kΩ
I=VR
I=2.4713100
I≈0.188mA

Circuit 1:

Explanation:
The simulation shows that when a 5 V peak-to-peak, 100 Hz sinusoidal signal is applied to a series resistor circuit, the voltage divides across the resistors according to their resistance values. The oscilloscope displays sinusoidal waveforms at different points in the circuit with the same frequency and phase but different amplitudes. This verifies the voltage divider rule, as the voltage across each resistor is proportional to its resistance.

Given values:
• Frequency = 100 Hz
• Input Voltage = 5 V peak-to-peak
• R1=5.1 kΩ
• R2=3.3 kΩ

In this circuit, we applied an AC signal using a signal generator and observed the voltage across each resistor using an oscilloscope. Channel A shows the input signal, while other channels show the voltage across R1and R2. The waveforms maintain the same shape but their amplitudes decrease due to voltage drops across resistors. This confirms that the circuit follows Ohm’s law and voltage division.

Circuit 2:

The simulation confirms that when a 7 V peak-to-peak, 200 Hz input signal is applied to a series resistor network, the voltage divides across the resistors based on their resistance values. The oscilloscope shows identical sinusoidal waveforms at different nodes with reduced amplitudes, verifying the voltage divider rule.

Given values:
• Frequency = 200 Hz
• Input Voltage = 7 V peak-to-peak
• R1=4.7 kΩ
• R2=3.3 kΩ
• R3=5.1 kΩ

In this circuit, three resistors are connected in series and supplied with an AC signal from a signal generator. The oscilloscope is used to observe voltage at different points. Channel A shows the input signal, Channel B shows the voltage after R1, Channel C shows the voltage after R1and R2, and Channel D shows the voltage after all resistors. The waveform shape remains sinusoidal, but the amplitude decreases at each stage due to voltage drops. This demonstrates practical verification of voltage division in series circuits.