Circuit Analysis (Lab4)

Experiment 4:  Implementation of Basic Parallel Circuit

Name:Muhammad Zuhaib	                         Registration No: 2025-CE-120

Date:   30 Mar. 26 	    Grade and Signature: ………………………..

| CLO2 CLO3 | Use with identification the circuit components, breadboards, multimeters, power supplies, signal generators, and oscilloscopes. Make measurements in their own constructed electric circuits. |  |  |  |
| --- | --- | --- | --- | --- |
| Psychomotor/Affective | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) |
| Report Marks (3) |  |  | Total marks (10) |  |

________________________________________________________________________________

Objectives: To make measurements in your own constructed parallel circuits.

Equipment required:

Power Supply, Breadboard, Connecting wires, Resistances, Multimeter.

# How to make resistance measurements

Resistance is measured with the circuit’s power turned off. The ohmmeter sends its own current through the unknown resistance and then measures that current to provide a resistance value readout.

Resistance measurement procedures

Follow the steps below to measure resistance:

- Turn off the power to the circuit. Remove or isolate the component to be tested.

- Plug the test probes into the appropriate probe jacks. Note that the jacks used may be the same ones used to measure volts.

- Select the ohms function by turning the function switch to ohms. Start with the lowest setting.

- Touch the probes together to check the leads, connections and battery life. The meter should display zero or a very small amount of resistance for the test leads. With the leads apart, the meter should display OL or I, depending on the manufacturer.

- Connect the tips of the probes across the break in the component or portion of the circuit for which you want to determine resistance. If you get an OL (over limit), move to the next level.

- View the reading on the display unit. Be sure to note the unit of measurement.

- After all the resistance readings are complete, turn the DMM off to prevent the battery from draining

Figure 1 Using a DMM to measure resistance

# How to make voltage measurements

Voltage measurements are very easy to make with a DMM. On meters with manual range selection, start with a value one setting higher than expected. An auto range DMM will automatically select the range based on the voltage present. Figure 4 shows the process.

Note:

1/ 1000 V=1mV

1000 V=1kV

Follow these steps to measure voltage:

- Select the DC or AC volts function by turning the function switch to DC or AC volts.

- Plug the test probes into the appropriate probe jacks.

- Connect the tips of the probes across the source or load.

- View the reading on the display unit. Be sure to note the unit of measurement. If you are testing DC voltage and a negative sign appears in the display, the polarity of your probes is incorrect and needs to be reversed.

Figure 2 Using a DMM to measure voltage

# How to make Current measurements

Follow the steps below to measure current:

- Turn off the power to the circuit that will be measured.

- Open the circuit by disconnecting or unsoldering a connection at a point where you wish to measure current.

- Select the DC or AC amps function by turning the function switch to DC or AC amps.

- Plug the test probes into the appropriate probe jacks. Note that the jacks used may not be the same ones used to measure volts.

- Connect the tips of the probes across the break in the circuit so that the current to be measured flows through the meter. Note that this is a series connection. Never connect the ammeter in parallel with the source or load, as this will cause a short circuit and damage the meter.

- Turn the circuit power back on.

- View the reading on the display unit. Be sure to note the unit of measurement.

- Switch power off and disconnect meter leads from the circuit.

- If testing is finished at this test point, restore the circuit by reclosing the connection. When current measurements are complete, turn the function switch to the OFF position and remove the test leads.

Figure 3 Using a DMM to measure current

Lab Tasks:

      - Measure the current across R1, R2, R3 and Voltage between node 2 and 7.

Practical Values:

R1 Current

0.9 mA

R2 Current

4.5 mA

R3 Current

9.0 mA

Voltage

9 V

Theoretical Values:

R1 Current

0.9 mA

R2 Current

4.5 mA

R3 Current

9.0 mA

Voltage

9 V

      - Measure the current across R1, R2, R3 and Voltage between node a and e.

Practical Values:

R1 Current

8.99 mA

R2 Current

3 mA

R3 Current

1.8 mA

R4 Current

0.9 mA

Voltage

8.99 V

Theoretical Values:

| R1 Current | R2 Current | R3 Current | R4 Current | Voltage |
| --- | --- | --- | --- | --- |
| 8.99 mA | 3 mA | 1.8 mA | 0.9 mA | 8.99 V |

Post Lab Questions:

1. Why is the value of voltage across each resistor in a parallel circuit the same?

In a parallel circuit, all components are connected directly across the same two nodes of the power source. This configuration ensures that each resistor experiences the full potential difference of the source, independent of the other resistors. Mathematically:

VR1=VR2=VR3=Vsource

This is a fundamental property of parallel circuits and is independent of the resistor values.

2. When parallel resistors are of three different values, which has the greatest power loss?

Power dissipated in a resistor is:

P=I2R=V2R

In a parallel circuit:

- Voltage across each resistor is the same (V = source voltage)

- Therefore, power loss is inversely proportional to resistance:

P=V2R  ⟹  smaller resistance → larger power dissipation

Conclusion: Among R1, R2, R3 with different values, the resistor with the lowest resistance has the greatest power loss.

3. How does the current across any branch of a parallel circuit vary?

In a parallel circuit, the current through a branch is inversely proportional to its resistance, given by Ohm’s Law:

Ibranch=VsourceRbranch

- Branch with smaller resistance → higher current

- Branch with larger resistance → lower current

The sum of all branch currents equals the total current drawn from the source:

Itotal=i=1nIi

4. What happens to total resistance in a circuit with parallel resistors if one of them opens (i.e., removed)?

Total resistance of a parallel circuit is given by:

1Rtotal=1R1+1R2+1R3+…

- Removing (opening) one resistor removes its branch from the parallel network.

- This increases total resistance of the remaining parallel network because there are fewer paths for current.

- Consequently, total current drawn from the source decreases, but voltage across remaining resistors remains constant.

Explaination:

In Lab 4, we implemented a parallel resistor network using a DC voltage source connected to four resistors of different values: R1 = 100 Ω, R2 = 3 kΩ, R3 = 5 kΩ, and R4 = 10 kΩ. The top ends of all resistors were joined at a common node connected to the positive terminal of the source, while the bottom ends were joined at another common node connected to the negative terminal. This ensured that each resistor experienced the same voltage as the source.

To measure the current through each resistor, we temporarily inserted a virtual ammeter in series with the branch under measurement, while keeping the other branches connected to the source. This allowed us to individually determine the branch currents without altering the parallel connections. The voltage across each resistor was measured using a voltmeter connected across the common nodes, confirming that all resistors share the same voltage in a parallel configuration.

Using the measured currents and known resistor values, we calculated the equivalent resistance of the parallel network and verified that the total current drawn from the source equals the sum of branch currents. The results also illustrated that resistors with lower resistance draw higher currents, demonstrating the distribution of current in parallel circuits.

This simulation reinforced the theoretical principles of parallel circuits: equal voltage across all branches, current division based on resistance, and reduced total resistance compared to the smallest individual resistor.