Circuit Analysis (Lab # 6)

Experiment 6:  VERIFICATION OF KIRCHHOFFS LAWS

Name: …………………………….  	 	            Registration No: …………………….

Date:   …………………………….  	 	Grade and Signature: ………………………

| CLO2 CLO3 | Use with identification the circuit components, breadboards, multimeters, power supplies, signal generators, and oscilloscopes. Make measurements in their own constructed electric circuits. |  |  |  |
| --- | --- | --- | --- | --- |
| Psychomotor/Affective | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) |
| Report Marks (3) |  |  | Total marks (10) |  |

# Objective

To verify Kirchhoff’s current law and Kirchhoff’s voltage law for the given Circuit.

Apparatus Required

| Sr. no | Apparatus | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply | Standard | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter | Standard | 1 |
| 5 | Breadboard & Wires | Standard | As Required |
| 6 | Resistors | 330ohm, 220ohm 1kohm | 3 |

Description

KCL: The algebraic sum of the currents meeting at a node is equal to zero.

KVL: In any closed path / mesh, the algebraic sum of all the voltages is zero.

Precautions:

- Voltage control knob should be kept at minimum position.

- Current control knob of RPS should be kept at maximum position.

Procedure for KCL:

- Give the connections as per the circuit diagram.

- Set a value in RPS.

- Note down the corresponding readings using multimeter

- Repeat the same for different voltages

Circuit - KCL

KCL - Practical Values:

| Sl. No. | Voltage E |  | Current |  | I1 = I2 + I3 |
| --- | --- | --- | --- | --- | --- |
|  |  | I1 | I2 | I3 |  |
|  | Volts | mA | mA | mA | mA |
| 1 | 5 | 5.68 | 2.56 | 3.12 | 11.36 |
| 2 | 15 | 17.1 | 7.68 | 9.37 | 34.15 |
| 3 | 25 | 28.4 | 12.8 | 15.6 | 56.8 |

Procedure for KVL:

- Give the connections as per the circuit diagram.

- Set a value in RPS.

- Note all the voltage reading

- Repeat the same for different voltages

## Circuit - KVL

## KVL - Practical Values

| Sl. No. | RPS |  | Voltage |  | E1 = V1 + V2 |
| --- | --- | --- | --- | --- | --- |
|  | E1 | V1 | V2 | V3 |  |
|  | V | V | V | V | V |
| 1 | 5 | 3.23 | 1.77 | 1.77 | 6.77 |
| 2 | 10 | 9.70 | 5.30 | 5.30 | 20.30 |
| 3 | 15 | 16.2 | 8.83 | 8.83 | 33.86 |

Result:

Thus, Kirchhoff’s voltage law and Kirchhoff’s current law are verified practically by performing this experiment.

## Post Lab Questions

- Define Kirchhoff’s current law.

Kirchhoff’s Current Law states that the algebraic sum of all currents entering and leaving a node is equal to zero. In other words, the total current entering a junction is equal to the total current leaving that junction. This law is based on the principle of conservation of charge, which states that charge cannot accumulate at a node.

- Define Kirchhoff’s voltage law.

Kirchhoff’s Voltage Law states that the algebraic sum of all voltages in any closed loop or mesh of a circuit is equal to zero. This means that the total voltage supplied in a loop is equal to the total voltage drop across the components in that loop. This law is based on the principle of conservation of energy.

- What are the applications of regulated power supply?

A regulated power supply is used to provide a constant and stable output voltage regardless of variations in input voltage or load conditions. It is commonly used in electronic circuits, laboratories, communication systems, computers, and testing equipment where a steady voltage is required for proper operation.

- What are the limitations of KCL?

Kirchhoff’s Current Law assumes that no charge is stored at the node, which may not be accurate at very high frequencies where capacitive effects become significant. It also does not directly account for electromagnetic effects such as displacement current. Therefore, KCL is mainly applicable to low-frequency and steady-state circuits.